# xxxxxx_PTD

Programming and Test instructions

Contents

Revision History	2

Summary	3

Document Parameters	3

Program and Configure	4

Verify	6

# Revision History

| Revision | Date | By | Changelog |
| --- | --- | --- | --- |
| A | 2024-11-27 | Phil Spindler | Initial Release |

# Summary

## Document Parameters

The following parameters are used in other parts of this document and are likely to change over the course of the product life (e.g. when a firmware update is issued).

| Parameter | Value |
| --- | --- |
| AssyPartNumber | 123456 |

# Program and Configure

| Supporting Information |  |
| --- | --- |
| Purpose | Program board components with necessary firmware for correct device operation, verify firmware. |
| Prerequisites | Fully assembled circuit card assembly |
| Equipment | - STLINK v2 - Power source for powering board (5-9V) |
| Relevant Pinouts | [J3] Programming/Debug Through-Hole Test Points Pin; Signal; STLINK Pin. 1; 3.3V; 1. 2; SWCLK; 9. 3; SWDIO; 7. 4; #RESET; 15. 5; GND; 2. 6; UART RX; -. 7; UART TX; -. 8; #RUN/BOOT; -. 9; DEBUG MODE; -. [J4] Power Through-Hole Test Points Pin; Signal. 1; 5-9VDC. 2; GND. |

| Process |  |  |
| --- | --- | --- |
| Note |  | n/a |
| 1 | Setup | Connect PCBA to programming and power interface described in “Pinouts” (either via connectors or test pads) using STLINK v2. Note pins in J3 and J4 are through-hole test pads with a pin spacing of 2.54mm and a hole size of 1mm. |
| 2 | Power | Apply power to the device via J4 |
| 3 | Program | Program device using the following command. Program_the_thing.exe -i <FIRMWARE FILE PENDING> -s {AssyPartNumber} |
| 4 | Check Result | Programming utility must report success with no errors. |

# Verify

| Supporting Information |  |
| --- | --- |
| Purpose | Verify board functions correctly |
| Prerequisites | Fully Assembled, registered, programmed & configured circuit card assembly. |
| Equipment | - Power source for DUT power (5V -> 9V) - 3.3V UART interface for J3 UART pins |
| Relevant Pinouts | [J3] Programming/Debug Through-Hole Test Points Pin; Signal; Host UART Interface. 1; 3.3V; -. 2; SWCLK; -. 3; SWDIO; -. 4; #RESET; -. 5; GND; GND. 6; UART RX; RX. 7; UART TX; TX. 8; #RUN/BOOT; -. 9; DEBUG MODE; -. [J4] Power Through-Hole Test Points Pin; Signal. 1; 5-9VDC. 2; GND. |
| UART Configuration | Baud Rate: 921600 Data Bits: 8 Parity Bits: None Stop Bits: 1 |

| Process |  |  |
| --- | --- | --- |
| 1 | Setup | Connect board to programming and power interface described in “Pinouts” (either via connectors or test pads) using STLINK v2. Note pins in J3 and J4 are through-hole test pads with a pin spacing of 2.54mm and a hole size of 1mm. |
| 2 | Power | Apply power to the device via J4 |
| 3 | Verify | On boot, the device will issue information over the UART interface. |